| NFPA | HMIS |
| --- | --- |
| Health-2 Fire-0 Reac-0 | Health-3 Fire-0 Reac-0 |

DATE OF ISSUE- 6/1/00

I. IDENTIFICATION AND EMERGENCY INFORMATION

| PRODUCT NAME: | ACID POWER |
| --- | --- |
| PRODUCT CODE: | 050 |
| CHEMICAL NAME: | MIXTURE |
| CAS NUMBER: | N/A |
| PRODUCT APPEARANCE AND ODOR: | LIGHT GREEN COLOR WITH NO ODOR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| PHOSPHORIC ACID | 7664-38-2 | <30% |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | FLUSH WITH WATER FOR 15 MINUTES-IF IRRITATION PERSISTS OBTAIN MEDICAL ATTENTION |
| --- | --- |
| SKIN CONTACT | FLUSH WITH WATER-IF IRRITATION PERSISTS OBTAIN MEDICAL ATTENTION |
| INHALATION | MOVE TO FRESH AIR-IF BREATHING IS DIFFICULT OR DISCOMFORT PERSISTS OBTAIN MEDICAL ATTENTION |
| INGESTION | RINSE MOUTH WITH WATER-INGEST LARGE AMOUNTS OF WATER TO CAUSE DILUTION-DO NOT CAUSE VOMITING !!! CONTACT PHYSICIAN IMMEDIATELY |
| # IV. FIRE AND EXPLOSION DATA |  |

| FLASH POINT (MINIMUM): | NON COMBUSTIBLE (NOTE: ALTHOUGH PHOSPHORIC ACID IS NOT COMBUSTIBLE, IT CAN REACT WITH METALS TO LIBERATE HYDROGEN, A FLAMMABLE GAS) |
| --- | --- |
| AUTOINGNITION TEMPERATURE: | NON COMBUSTIBLE |
| HANDLING PRECAUTIONS: | USE GOGGLES AND/OR FACE SHIELDS-RUBBER GLOVES AND BOOTS WHERE CONTACT IS EXPECTED |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | USE FULL PROTECTIVE CLOTHING AND SELF CONTAINED BREATHING APPARATUS |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | AT FIRE TEMPERATURES, ACID POWER WILL EMIT TOXIC PHOSPHORUS OXIDE FUMES |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER- DISPOSE OF CONTAINER IN CONFORMANCE WITH FEDERAL, STATE, AND LOCAL REGULATIONS |

V. HEALTH AND HAZARD INFORMATION

| VARIABILITY AMONG INDIVIDUALS: | NONE KNOWN |
| --- | --- |
| EFFECTS OF OVEREXPOSURE: (SIGNS AND SYMPTOMS OF EXPOSURE | SLIGHTLY TOXIC WITH REPEATED INHALATION OR INGESTION-CAUSES BURNS TO EXPOSED TISSUES |

NATURE OF HAZARD AND TOXICITY INFORMATION:

| ACUTE ORAL TOXICITY | 50 TO 500 mg/kg |
| --- | --- |
| SKIN IRRITATION | MILD PRIMARY SKIN IRRITANT |
| primary irritation score (index) | 1.12 |
| EYE IRRITATION | EYE IRRITANT |
| mean maximum score | 75.3 |
| VI. PHYSICAL DATA |  |

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | 130C - 158C |
| --- | --- |
| SPECIFIC GRAVITY | 1.2 |
| pH | <1 |
| VAPOR PRESSURE | 0.3 mm Hg at 20C |
| PERCENT VOLATILE BY WEIGHT | 64% @ 100 o C / 2 HOURS |
| SOLUBILITY IN WATER @ 1 ATM AND 25C | COMPLETE |

VII. REACTIVITY

CONTACT WITH REACTIVE METALS (e.g. MILD STEEL-ALUMINUM) MAY PRODUCE FLAMMABLE/EXPLOSIVE HYDROGEN-ACID MIXTURES REACT VIOLENTLY WITH STRONG ALKALI (BASES)

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: DIKE AND SALVAGE OR NEUTRALIZE WITH LIME OR SODA ASH AND DISPOSE INTO TREATMENT SYSTEM IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL LAWS

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | ADEQUATE TO MEET MAXIMUM EXPOSURE LIMIT FOR ACID MIST OF 1 mg/m3 FOR 8 HOURS |
| --- | --- |
| RESPIRATORY PROTECTION: | FOR SEVERE MIST USE NIOSH APPROVED ACID RESPIRATOR |
| PROTECTIVE GLOVES: | RUBBER, NEOPRENE OR OTHER ACID RESISTANT MATERIAL |
| EYE PROTECTION: | CHEMICAL GOGGLES |
| OTHER PROTECTIVE EQUIPMENT: | RUBBER APRON AND BOOTS WHERE CONTACT IS LIKELY |
| PERSONAL HYGIENE: | WASH AFTER HANDLING-CLEAN CONTAMINATED GARMENTS BEFORE USING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | CLASSIFIED AS A CORROSIVE-PROTECT AGAINST PHYSICAL DAMAGE |
| --- | --- |
| DOT IDENTIFICATION NUMBER | NA 1760 |
| SHIPPING DESCRIPTION | COMPOUNDS, CLEANING LIQUID, (PHOSPHORIC ACID), 8,NA 1760, PG III, ITEM# 48580, SUB 3 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.