| NFPA | HMIS |
| --- | --- |
| Health-3 Fire-0 Reac-2 | Health-3 Fire-0 Reac-2 |

DATE OF ISSUE 6/1/00

I. IDENTIFICATION AND EMERGENCY INFORMATION

| PRODUCT NAME: | ALKA-DET® 2 |
| --- | --- |
| PRODUCT CODE: | 192 |
| CHEMICAL NAME: | MIXTURE |
| CAS NUMBER: | N/A |
| PRODUCT APPEARANCE AND ODOR: | CLEAR YELLOW LIQUID CHLORINE ODOR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| POTASSIUM HYDROXIDE | 1310-58-3 | <15% |
| SODIUM HYPOCHLORITE | 7681-52-9 | 3% |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | FLUSH WITH WATER FOR 15 MINUTES - GET MEDICAL ATTENTION |
| --- | --- |
| SKIN CONTACT | WASH WITH WATER - REMOVE CONTAMINATED CLOTHING |
| INHALATION | MOVE TO FRESH AIR - GET MEDICAL ATTENTION |
| INGESTION | DRINK PLENTY OF WATER OR FRUIT JUICE - DO NOT INDUCE VOMITING - GET MEDICAL ATTENTION |

IV. FIRE AND EXPLOSION DATA

| FLASH POINT (MINIMUM): | NON FLAMMABLE |
| --- | --- |
| HANDLING PRECAUTIONS: | DO NOT PERMIT EMPLOYEES TO HANDLE ALKA-DET® 2 WITHOUT PROPER PROTECTIVE EQUIPMENT |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | WEAR CLOTHING AND EQUIPMENT TO AVOID BODILY CONTACT - THIS MATERIAL MAY BE CORROSIVE TO HUMAN TISSUE - EXTINGUISHING MEDIA SHOULD BE SUITABLE FOR SURROUNDING FIRE |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | RELEASE OF CHLORINE VAPORS |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER - DISPOSE OF IN CONFORMANCE WITH FEDERAL, STATE, AND LOCAL REGULATIONS |

V. HEALTH AND HAZARD INFORMATION

| VARIABILITY AMONG INDIVIDUALS: | NONE KNOWN |
| --- | --- |
| EFFECTS OF OVEREXPOSURE: (SIGNS AND SYMPTOMS OF EXPOSURE | CONTACT WITH CONCENTRATED SOLUTION MAY CAUSE SERIOUS BURNS TO HUMAN TISSUE |

NATURE OF HAZARD AND TOXICITY INFORMATION:

| ACUTE ORAL TOXICITY | LESS THAN 5 ml/kg - MAY CAUSE DAMAGE TO THROAT AREA |
| --- | --- |
| SKIN IRRITATION | PRIMARY SKIN IRRITANT |
| primary irritation score (index) | 5.0 |
| EYE IRRITATION | POSITIVE EYE IRRITANT |

VI. PHYSICAL DATA

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | 100 o C |
| --- | --- |
| SPECIFIC GRAVITY | 1.2 |
| pH | 13.5 |
| PERCENT VOLATILE BY WEIGHT | 76% @ 100oC / 2 HOURS |
| EVAPORATION RATE @ 1 ATM AND 25C | LIKE WATER |
| SOLUBILITY IN WATER @ 1 ATM AND 25C | COMPLETE |

VII. REACTIVITY

DO NOT ALLOW CONTACT WITH ACIDS

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:  PROTECTIVE CLOTHING AND EQUIPMENT MUST BE WORN BY CLEAN UP PERSONNEL - CONTAIN SPILLAGE OR LEAKAGE IN SUITABLE CONTAINERS OR CONTAIN IN A HOLDING AREA - DO NOT ALLOW DRAINAGE TO SEWERS, STREAMS OR STORM CONDUITS - RECOVER WITH VACUUM EQUIPMENT SUCH AS A SEPTIC TANK TRUCK AND FLUSH WITH WATER

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | PROVIDE ADEQUATE VENTILATION |
| --- | --- |
| RESPIRATORY PROTECTION: | NONE NORMALLY REQUIRED - IF SPRAYING OR MISTING WEAR APPROVED NIOSH MIST RESPIRATOR |
| PROTECTIVE GLOVES: | RUBBER LATEX OR PLASTIC |
| EYE PROTECTION: | SPLASH GOGGLES OR FACE SHIELD |
| OTHER PROTECTIVE EQUIPMENT: | RUBBER BOOTS AND APRON |
| WORK PRACTICES / ENGINEERING CONTROL: | RUBBERS OVER LEATHER IS NOT RECOMMENDED |
| PERSONAL HYGIENE: | CLEAN CONTAMINATED GARMENTS BEFORE REUSING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | AVOID OVER HEATING OR FREEZING - STORE IN A COOL, WELL VENTILATED AREA - SPILLED MATERIAL IS SLIPPERY - CLASSIFIED AS A CORROSIVE |
| --- | --- |
| DOT IDENTIFICATION NUMBER | NA 1760 |
| SHIPPING DESCRIPTION | COMPOUNDS, CLEANING LIQUID, (POTASSIUM HYDROXIDE), 8, NA 1760, PG II, ITEM # 48580, SUB 3 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.