| NFPA | HMIS |
| --- | --- |
| Health-1 Fire-0 Reac-0 | Health-1 Fire-0 Reac-0 |

DATE OF ISSUE-6/1/00

I. IDENTIFICATION AND EMERGENCY INFORMATION

| PRODUCT NAME: | CRYSTALIT™ |
| --- | --- |
| PRODUCT CODE: | 480 |
| CHEMICAL NAME: | ABSORBENT POLYMER |
| CAS NUMBER: | N/A |
| PRODUCT APPEARANCE AND ODOR: | WHITE OPAQUE - NO ODOR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| NO HAZARDOUS COMPONENTS |  |  |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | FLUSH WITH WATER FOR 15 MINUTES |
| --- | --- |
| SKIN CONTACT | WASH AFFECTED AREA WITH SOAP AND WATER |
| INHALATION | MOVE TO FRESH AIR |
| INGESTION | PROMPTLY DRINK LARGE AMOUNTS OF MILK, EGG WHITES, GELATIN SOLUTION OR WATER - AVOID ALCOHOL - CALL PHYSICIAN IMMEDIATELY |

IV. FIRE AND EXPLOSION DATA

| FLASH POINT (MINIMUM): | NOT APPLICABLE |
| --- | --- |
| HANDLING PRECAUTIONS: | N/A |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | THIS MATERIAL WILL NOT BURN - USE EXTINGUISHING MEDIA APPROPRIATE FOR THE SURROUNDING FIRE - WEAR SELF CONTAINED POSITIVE PRESSURE BREATHING APPARATUS |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | NONE KNOWN |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER - DISPOSE OF IN CONFORMANCE WITH FEDERAL, STATE AND LOCAL REGULATIONS |

V. HEALTH AND HAZARD INFORMATION

| VARIABILITY AMONG INDIVIDUALS: | NONE KNOWN |
| --- | --- |
| EFFECTS OF OVEREXPOSURE: (SIGNS AND SYMPTOMS OF EXPOSURE | NONE KNOWN |

NATURE OF HAZARD AND TOXICITY INFORMATION:

THIS MATERIAL PRODUCED NO EYE IRRITATION AND MILD SKIN IRRITATION IN RABBIT TESTING - NO SENSITIZATION WAS PRODUCED IN A REPEATED INSULT HUMAN PATCH TEST - NO MORTALITY WAS OBSERVED IN AN ACUTE DUST INHALATION STUDY IN RATS AT 200 mg/m³ - THIS MATERIAL WAS NOT MUTAGENIC IN AMES TEST

VI. PHYSICAL DATA

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | N/A |
| --- | --- |
| SPECIFIC GRAVITY | N/A |
| pH | N/A |
| PERCENT VOLATILE BY VOLUME | 13% AS H2O |
| SOLUBILITY IN WATER @ 1 ATM AND 25°C | SWELLS (LOW) |

VII. REACTIVITY

SEE SECTION IV

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS SPILLED OR RELEASED: SPILLED MATERIAL BECOMES VERY SLIPPERY WHEN WET - FLUSH THE AREA THOROUGHLY WITH WATER AND SCRUB TO REMOVE RESIDUE - IF SURFACE REMAINS SLICK, APPLY MORE DRY SWEEPING COMPOUND

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | LOCAL EXHAUST |
| --- | --- |
| RESPIRATORY PROTECTION: | N/A |
| PROTECTIVE GLOVES: | RUBBER OR NEOPRENE |
| EYE PROTECTION: | CHEMICAL GOGGLES |
| OTHER PROTECTIVE EQUIPMENT: | FACE SHIELD |
| PERSONAL HYGIENE: | WASH AFTER HANDLING - CLEAN CONTAMINATED GARMENTS BEFORE REUSING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | AVOID OVERHEATING OR FREEZING -STORE IN COOL WELL VENTILATED AREA - SPILLED MATERIAL IS SLIPPERY - THIS PRODUCT IS CLASSIFIED AS NON-HAZARDOUS |
| --- | --- |
| DOT IDENTIFICATION NUMBER | N/A |
| SHIPPING DESCRIPTION | PLASTIC MATERIAL, OT EXPANDED, ITEM # 156200 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.