| NFPA | HMIS |
| --- | --- |
| Health- 1 Fire- 1 Reac-0 | Health-1 Fire-1 Reac-0 |

DATE OF ISSUE- 6/1/00

I. IDENTIFICATION AND EMERGENCY INFORMATION

| PRODUCT NAME: | FOAM BOOSTER |
| --- | --- |
| PRODUCT CODE: | 400 |
| CHEMICAL NAME: | MIXTURE |
| CAS NUMBER: | N/A |
| PRODUCT APPEARANCE AND ODOR: | CLEAR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| Anionic Surfactant | Proprietary | 13% |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | Will cause Irritation. Flush with running water for 15 minutes. Seek medical assistance if irritation persists. |
| --- | --- |
| SKIN CONTACT | Prolonged and repeated contact will cause deoiling of the skin resulting in inflammation, rashes or dermatitis. Flush contaminated area thoroughly with water. If irritation persists seek medical assistance. |
| INHALATION | Move to fresh air |
| INGESTION | Drink several glasses of water. Do not induce vomiting. Call a physician. |

IV. FIRE AND EXPLOSION DATA

| FLASH POINT (MINIMUM): | N/A |
| --- | --- |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | For surrounding fire. |

V. HEALTH AND HAZARD INFORMATION

NATURE OF HAZARD AND TOXICITY INFORMATION:

| ACUTE ORAL TOXICITY | LD50: 5 g / Kg |
| --- | --- |
| SKIN IRRITATION | 2 g / 24 H MOD |
| EYE IRRITATION | 1 mg / 24 H |

VI. PHYSICAL DATA

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | ND |
| --- | --- |
| SPECIFIC GRAVITY | 1.02 |
| pH | 5.6 |
| PERCENT VOLATILE BY VOLUME | 87.5 % @ 100oC / 2 HOURS |
| EVAPORATION RATE @ 1 ATM AND 25C | LIKE WATER |
| SOLUBILITY IN WATER @ 1 ATM AND 25C | COMPLETE |

VII. REACTIVITY

| STABILITY: | STABLE |
| --- | --- |
| CONDITIONS TO AVOID: | NONE |
| INCOMPATIBILITY: | NONE |
| HAZARDOUS DECOMPOSITION: | NONE |
| HAZARDOUS POLYMERIZATION: | WILL NOT OCCUR |

VIII. SPILL OR LEAK PROCEDURES

Material is readily biodegradable in waste water treatment systems. For small spills, wash away with water. The washings should be discharged to a plant or municipal water treatment system. Avoid direct discharge to natural waterways.

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | NONE REQUIRED |
| --- | --- |
| RESPIRATORY PROTECTION: | NONE REQUIRED |
| PROTECTIVE GLOVES: | CHEMICAL RESISTANT |
| EYE PROTECTION: | SAFETY GLASSES OR GOGGLES OR FULL FACESHIELD |
| PERSONAL HYGIENE: | WASH AFTER HANDLING. CLEAN GARMENTS BEFORE REUSING. |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | NOT REGULATED IN DRUMS OR CONTAINERS UNDER 3,500 lbs. |
| --- | --- |
| DOT IDENTIFICATION NUMBER |  |
| SHIPPING DESCRIPTION |  |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. I N ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.