| NFPA | HMIS |
| --- | --- |
| Health-2 Fire-0 Reac-1 | Health-2 Fire-0 Reac-1 |

DATE OF ISSUE-6/1/00

I. IDENTIFICATION AND EMERGENCY INFORMATION

| PRODUCT NAME: | HYPOCHLOR® |
| --- | --- |
| PRODUCT CODE: | 070 |
| CHEMICAL NAME: | SODIUM HYPOCHLORITE |
| CAS NUMBER: | 7681-52-9 |
| PRODUCT APPEARANCE AND ODOR: | YELLOW-GREEN LIQUID |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| SODIUM HYPOCHLORITE | 7681-52-9 UN 1791 | 12.5% |
| SODIUM HYDROXIDE | 1310-73-2 UN 1823 | 0.8% |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | FLUSH WITH WATER FOR 15 MINUTES - GET MEDICAL ATTENTION |
| --- | --- |
| SKIN CONTACT | REMOVE SOAK CLOTHES; RINSE WITH SOAP AND WATER FOR AT LEAST 5 MINUTES |
| INHALATION | MOVE TO FRESH AIR - CALL PHYSICIAN IF EXPOSURE IS SEVERE |
| INGESTION | IF CONSCIOUS, DRINK LARGE QUANTITIES OF MILK OR GELATIN SOLUTION; IF THESE ARE NOT AVAILABLE, DRINK LARGE QUANTITIES OF WATER - DO NOT GIVE VINEGAR OR OTHER ACIDS - DO NOT INDUCE VOMITING - GET PROMPT MEDICAL ATTENTION |
| IV. FIRE AND EXPLOSION DATA |  |

| FLASH POINT (MINIMUM): | NOT ESTABLISHED (NOTE: MATERIAL IS A STRONG OXIDIZER - CONTACT WITH COMBUSTIBLES MAY INITIATE OR PROMOTE COMBUSTION) |
| --- | --- |
| HANDLING PRECAUTIONS: | USE GOGGLES AND/OR FACE SHIELDS - RUBBER GLOVES AND BOOTS WERE LIQUID CONTACT IS EXPECTED |
| FLAMMABLE OR EXPLOSIVE LIMITS (APPROX. % BY VOLUME IN AIR): | NOT ESTABLISHED |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | FLOOD WITH WATER OR CARBON DIOXIDE (CO2) |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | MAY PRODUCE HYDROCHLORIC ACID AND/OR CHLORINE GAS |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER - DISPOSE OF CONTAINER IN CONFORMANCE WITH FEDERAL, STATE, AND LOCAL REGULATIONS |
| V. HEALTH AND HAZARD INFORMATION |  |

| VARIABILITY AMONG INDIVIDUALS: | NONE KNOWN |
| --- | --- |
| EFFECTS OF OVEREXPOSURE: (SIGNS AND SYMPTOMS OF EXPOSURE | EYES & SKIN: EXTREME IRRITANT, CORROSIVE INHALATION OR INGESTION: IRRITATING TO MUCOUS MEMBRANES - MOUTH, THROAT AND STOMACH PAIN - POSSIBLE ULCERATION |

NATURE OF HAZARD AND TOXICITY INFORMATION:

| ACUTE OVEREXPOSURE SWALLOWING | SEE INGESTION UNDER HEALTH AND HAZARD |
| --- | --- |
| SKIN CONTACT | IRRITANT, CORROSIVE |
| INHALATION | FUMES FROM SPILLS IRRITATING TO MUCOUS MEMBRANES |
| EYE CONTACT | EXTREME IRRITANT, CORROSIVE |
| CHRONIC OVEREXPOSURE | EYES: CAN CAUSE DAMAGE SKIN: CAN CAUSE DAMAGE - CHEMICAL BURN |

VI. PHYSICAL DATA

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT (760 mm Hg) | DECOMPOSES |
| --- | --- |
| SPECIFIC GRAVITY | 1.2 |
| pH | 12.5-13.5 |
| VAPOR PRESSURE | VARIABLE - WATER PLUS PRODUCTS OF DECOMPOSITION |
| PERCENT VOLATILE BY VOLUME | VARIABLE - WATER PLUS PRODUCTS OF DECOMPOSITION |
| SOLUBILITY IN WATER @ 1 ATM AND 25°C | COMPLETE |

VII. REACTIVITY

CONTACT WITH ACID RELEASES CHLORINE GAS - NATURAL DECOMPOSITION PRODUCT IS OXYGEN - THERMAL DECOMPOSITION OR BURNING CAN PRODUCE HYDROCHLORIC ACID

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: MATERIAL IS TOXIC TO FISH - CONTAIN IN DIKED AREA - NEUTRALIZE WITH SODIUM SULFITE, SODIUM BISULFITE OR FERROUS SALT SOLUTIONS - DILUTE COPIOUSLY WITH WATER - DISPOSE OF IN ACCORDANCE WITH FEDERAL, STATE, AND LOCAL LAWS

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | LOCAL EXHAUST |
| --- | --- |
| RESPIRATORY PROTECTION: | USE NIOSH CERTIFIED GAS MASK WITH CANISTER IF CHLORINE FUMES ARE PRESENT |
| PROTECTIVE GLOVES: | RUBBER |
| EYE PROTECTION: | CHEMICAL GOGGLES |
| OTHER PROTECTIVE EQUIPMENT: | USE RUBBER SPLASH APRON - SAFETY SHOWER & EYEWASH FOUNTAIN SHOULD BE LOCATED NEARBY |
| WORK PRACTICES / ENGINEERING CONTROL: | RUBBERS OVER LEATHER IS NOT RECOMMENDED |
| PERSONAL HYGIENE: | WASH AFTER HANDLING - CLEAN CONTAMINATED GARMENTS BEFORE REUSING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | STORE UPRIGHT IN A COOL, DRY AREA AWAY FROM DIRECT SUNLIGHT AND EXCESSIVE HEAT - DO NOT STORE NEAR OTHER INCOMPATIBLE SWIMMING POOL PRODUCTS SUCH AS CHLORINATED ISOCYRANURATES OR MURIATIC ACID |
| --- | --- |
| DOT IDENTIFICATION NUMBER | UN 1791 |
| SHIPPING DESCRIPTION | HYPOCHLORITE SOLUTION, 8, UN 1791, PG III, ITEM # 46380, SUB 1 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.