| NFPA | HMIS |
| --- | --- |
| Health-2 Fire-0 Reac-1 | Health-2 Fire-0 Reac-1 |
| DATE OF ISSUE-5/25/00 |  |
| I. IDENTIFICATION AND EMERGENCY INFORMATION |  |

| PRODUCT NAME: | QUATRICIDE® PV-15 |
| --- | --- |
| PRODUCT CODE: | 640 |
| CHEMICAL NAME: | GERMICIDAL DETERGENT |
| PRODUCT APPEARANCE AND ODOR: | DARK ORANGE- NO ODOR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| DIDECYL DIMETHYL AMMONIUM CHLORIDE | 7173-51-5 | 9.70% |
| N - ALKYL (C14 50%, C12 40%, C16 10%) DIMETHYL BENZYL AMMONIUM CHLORIDE | 8001-54-51 | 6.47% |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | IRRIGATE WITH FLOWING WATER FOR 15 MINUTES CALL A PHYSICIAN |
| --- | --- |
| SKIN CONTACT | FLUSH WITH FLOWING WATER FOR 15 MINUTES, THEN SOAP AND WATER - IF IRRITATION PERSIST, CALL A PHYSICIAN |
| INHALATION | MOVE TO FRESH AIR |
| INGESTION NOTE TO PHYSICIAN: | DO NOT INDUCE VOMITING OR GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON. PROMPTLY DRINK A LARGE QUANTITY OF MILK, EGG WHITES, GELATIN SOLUTION OR IF THESE ARE NOT AVAILABLE, DRINK LARGE QUANTITIES OF WATER - AVOID ALCOHOL - CALL A PHYSICIAN IMMEDIATELY PROBABLE MUCOSAL DAMAGE MAY CONTRAINDICATE THE USE OF GASTRIC LAVAGE. MEASURES AGAINST CIRCULATORY SHOCK, RESPIRATORY DEPRESSION, AND CONVULSION MAY BE NEEDED. |
| IV. FIRE AND EXPLOSION DATA |  |

| FLASH POINT (MINIMUM): | NON FLAMMABLE |
| --- | --- |
| HANDLING PRECAUTIONS: | USE GOGGLES AND/OR FACE SHIELD -RUBBER GLOVES AND BOOTS WHERE CONTACT IS EXPECTED |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | NONE KNOWN |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER. DISPOSE OF CONTAINER IN CONFORMANCE WITH FEDERAL, STATE AND LOCAL REGULATIONS |

V. HEALTH AND HAZARD INFORMATION

| VARIABILITY AMONG INDIVIDUALS: | NONE KNOWN |
| --- | --- |
| EFFECTS OF OVEREXPOSURE: | EYES: CAUSES IRREVERSIBLE EYE DAMAGE SKIN: SKIN BURNS |

NATURE OF HAZARD AND TOXICITY INFORMATION:

| ACUTE ORAL TOXICITY | 6.9 g/kg |
| --- | --- |
| ACUTE DERMAL IRRITATION | GREATER THAN 5g/kg |
| VI. PHYSICAL DATA |  |

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | 100°C |
| --- | --- |
| SPECIFIC GRAVITY | 1.00 |
| pH | 7.2-8.2 |
| VAPOR PRESSURE | 17.5 |
| VAPOR DENSITY (AIR=1) | < 1 |
| PERCENT VOLATILE BY WEIGHT | 67% @ 100oC / 2 HOURS |
| EVAPORATION RATE @ 1 ATM AND 25C (n-BUTYL ACETATE = 1) | 1 |
| SOLUBILITY IN WATER @ 1 ATM AND 25C | COMPLETE |

VII. REACTIVITY

CONDITIONS TO AVOID: HEAT, FIRE, IGNITION SOURCE

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: DO NOT DISCHARGE INTO LAKES, STREAMS, ESTUARIES, OCEANS OR OTHER WATERS, UNLESS IN ACCORDANCE WITH  THE REQUIREMENTS OF A NATIONAL POLLUTANT DISCHARGE ELIMINATION SYSTEM PERMIT AND THE PERMITING AUTHORITY HAS BEEN NOTIFIED IN WRITING PRIOR TO DISCHARGE. DO NOT DISCHARGE EFFLUENT CONTAINING THIS PRODUCT TO SEWER SYSTEM WITHOUT NOTIFYING THE LOCAL SEWAGE TREATMENT PLANT AUTHORITY.

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | LOCAL EXHAUST |
| --- | --- |
| RESPIRATORY PROTECTION: | IF VAPORS, FUMES OR DUST ARE PRESENT USE A NIOSH APPROVED RESPIRATOR |
| PROTECTIVE GLOVES: | LATEX OR VINYL |
| EYE PROTECTION: | CHEMICAL GOGGLES |
| OTHER PROTECTIVE EQUIPMENT: | FACE SHIELD |
| PERSONAL HYGIENE: | CLEAN CONTAMINATED GARMENTS BEFORE REUSING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | - STORE IN COOL AREA- AVOID OVERHEATING OR FREEZING |
| --- | --- |
| DOT IDENTIFICATION NUMBER | N/A |
| SHIPPING DESCRIPTION | DISINFECTANT, NOI, ITEM # 57100, SUB 3 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.