| NFPA | HMIS |
| --- | --- |
| Health-2 Fire-1 Reac-0 | Health-2 Fire-1 Reac-0 |

DATE OF ISSUE-5/30/00

I. IDENTIFICATION AND EMERGENCY INFORMATION

| PRODUCT NAME: | QUATRICIDE® |
| --- | --- |
| PRODUCT CODE: | 650 |
| CHEMICAL NAME: | SURFACTANT BLEND |
| CAS NUMBER: | MIXTURE |
| PRODUCT APPEARANCE AND ODOR: | LIGHT PINK COLOR - MINT ODOR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| n-Alkyl(60% C14, 30%C16, 5%C12, 5%C18) dimethyl benzyl ammonium chlorides | 68391-01-5 | 2.25% |
| Alky (68%C12, 32%C14) Dimetyl Eth. Ben. Amm. Chl. | 68956-79-6 | 2.25% |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | FLUSH EYES IMMEDIATELY WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES. CALL A PHYSICIAN. |
| --- | --- |
| SKIN CONTACT | WASH OFF AND REMOVE CONTAMINATED CLOTHING - DO NOT REUSE CONTAMINATED CLOTHING |
| INHALATION | IF DUST, VAPORS, OR MIST CAUSE DISTRESS - MOVE TO FRESH AIR - USE OXYGEN OR ARTIFICIAL RESPIRATION IF NEEDED |
| INGESTION | IF SWALLOWED,DRINK EGG WHITES OR GELATIN SOLUTION,IF THESE ARE NOT AVAILABLE, DRINK LARGE QUANTITIES OF WATER. AVOID ALCOHOL. CONSULT A PHYSICIAN . |
| IV. FIRE AND EXPLOSION DATA |  |

| FLASH POINT (MINIMUM): | 200°F |
| --- | --- |
| AUTOINGNITION TEMPERATURE: | NOT DETERMINED |
| HANDLING PRECAUTIONS: | USE IMPERVIOUS GLOVES AND OTHER PROTECTIVE EQUIPMENT TO PREVENT PROLONGED CONTACT |
| FLAMMABLE OR EXPLOSIVE LIMITS (APPROX. % BY VOLUME IN AIR): | NOT ESTABLISHED |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | CO2, DRY CHEMICAL, SYNTHETIC FOAM OR WATER - TYPE BC OR ABC EXTINGUISHERS |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | NONE KNOWN |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER. DISPOSE OF CONTAINER IN CONFORMANCE WITH FEDERAL, STATE AND LOCAL REGULATIONS |

V. HEALTH AND HAZARD INFORMATION

| VARIABILITY AMONG INDIVIDUALS: | NONE KNOWN |
| --- | --- |
| EFFECTS OF OVEREXPOSURE: (SIGNS AND SYMPTOMS OF EXPOSURE | EYES & SKIN: SEVERE IRRITATION INHALATION OR INGESTION: DROWSINESS, NAUSEA, LOSS OF MOTOR SKILLS OR DISORIENTATION |

NOTE TO PHYSICIAN:

PROBABLE MUCOUS DAMAGE MAY CONTRAINDICATE THE USE OF GASTRIC LAVAGE.

NATURE OF HAZARD AND TOXICITY INFORMATION:

| ACUTE ORAL TOXICITY | LD50 < 5.4 g/kg |
| --- | --- |
| ACUTE DERMAL IRRITATION | GREATER THAN 8g/kg |
| EYE IRRITATION | PRIMARY EYE IRRITANT |
| VI. PHYSICAL DATA |  |

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | 100°C |
| --- | --- |
| SPECIFIC GRAVITY | 1.038 |
| pH | 11 |
| VAPOR PRESSURE | LIKE WATER |
| PERCENT VOLATILE BY WEIGHT | 85% @ 100oC / 2 HOURS |
| EVAPORATION RATE @ 1 ATM AND 25C | SLOWER THAN ETHYL ETHER |
| SOLUBILITY IN WATER @ 1 ATM AND 25C | COMPLETE |

VII. REACTIVITY

STABILITY: STABLE

HAZARDOUS POLYMERIZATION: WILL NOT OCCUR AT AMBIENT

TEMPERATURES

MATERIALS TO AVOID: STRONG OXIDIZING AGENTS

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: CONTAIN ALL SPILLS AND LEAKS TO PREVENT DISCHARGE INTO THE ENVIRONMENT - VENTILATE AREA

SMALL SPILL: SOAK UP WITH ABSORBENT - SHOVEL INTO WASTE CONTAINERS - FLUSH AREA WITH WATER

LARGE SPILL: RECOVER SPILLED MATERIAL FOR REPROCESSING OR DISPOSAL

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | PROVIDE ADEQUATE VENTILATION TO KEEP HAZARDOUS INGREDIENTS BELOW THEIR TLV |
| --- | --- |
| RESPIRATORY PROTECTION: | IF VAPORS, FUMES OR DUST ARE PRESENT USE A MSHA OR NIOSH APPROVED RESPIRATOR, A FRESH AIR BREATHING APPARATUS OR A SELF CONTAINED BREATHING APPARATUS |
| PROTECTIVE GLOVES: | RUBBER |
| EYE PROTECTION: | WEAR GOGGLES AND FULL FACE SHIELD WHEN HANDLING TO AVOID CONTACT WITH FACE AND EYES |
| OTHER PROTECTIVE EQUIPMENT: | ANY OTHER PROTECTIVE EQUIPMENT TO PREVENT PROLONGED OR REPEATED CONTACT |
| PERSONAL HYGIENE: | WASH AFTER HANDLING - CLEAN CONTAMINATED GARMENTS BEFORE REUSING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | AVOID OVERHEATING OR FREEZING - STORE IN A COOL WELL VENTILATED AREA - SPILLED MATERIAL IS SLIPPERY - THIS PRODUCT IS CLASSIFIED AS NON- HAZARDOUS |
| --- | --- |
| DOT IDENTIFICATION NUMBER | N/A |
| SHIPPING DESCRIPTION | DISINFECTANT, NOI, ITEM # 57100, SUB 3 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.