| NFPA | HMIS |
| --- | --- |
| Health-2 Fire-0 Reac-1 | Health-2 Fire-0 Reac-1 |
| DATE OF ISSUE 5/25/00 |  |
| I. IDENTIFICATION AND EMERGENCY INFORMATION |  |

| PRODUCT NAME: | QUATRICIDE® TB |
| --- | --- |
| PRODUCT CODE: | 630 |
| CHEMICAL NAME: | NONIONIC / QUATERNARY BLEND |
| CAS NUMBER: | N/A |
| PRODUCT APPEARANCE AND ODOR: | CLEAR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| n-Alkyl (60%C14, 30%C16, 5% C12, 5% C18) dimethyl benzyl ammonium chlorides | 68391-01-5 | 0.105% |
| n-Alkyl (60%C12, 32%C14) dimethyl ethylbenzyl ammonium chlorides | 68956-79-6 | 0.105% |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | IRRIGATE WITH FLOWING WATER FOR 15 MINUTES CALL A PHYSICIAN |
| --- | --- |
| SKIN CONTACT | FLUSH WITH FLOWING WATER FOR 15 MINUTES - IF IRRITATION PERSIST, CALL A PHYSICIAN |
| INHALATION | MOVE TO FRESH AIR.USE OXYGEN OR ARTIFICIAL RESPIRATION, IF NEEDED |
| INGESTION | IF SWALLOWED, DRINK EGG WHITES OR GELATIN SOLUTION, OR IF THESE ARE NOT AVAILABLE, DRINK LARGE QUANTITIES OF WATER. CONSULT A PHYSICIAN IMMEDIATELY |
| NOTE TO PHYSICIAN: Probable mucosal damage may contraindicate the use of gastric lavage. |  |
| IV. FIRE AND EXPLOSION DATA |  |

| FLASH POINT (MINIMUM): | 200oF |
| --- | --- |
| HANDLING PRECAUTIONS: | SEVERELY IRRITATING TO SKIN & EYES |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | CO2, DRY CHEMICALS, SYNTHETIC FOAM OR WATER. TYPE BC OR ABC EXTINGUISHERS |
| SPECIAL FIRE FIGHTING PROCEDURES | USE SELF CONTAINED BREATHING APPARATUS FOR MAXIMUM RESPIRATORY PROTECTION. WATER MAY BE USED TO COOL CONTAINERS IF NOT RUPTURED OR LEAKING |
| HAZARDOUS DECOMPOSITION PRODUCTS: | NITROUS OXIDES, AMMONIACAL VAPORS |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER. DISPOSE OF CONTAINER IN CONFORMANCE WITH FEDERAL, STATE AND LOCAL REGULATIONS |

V. HEALTH HAZARD INFORMATION

EFFECTS OF EXPOSURE:  CONTACT WITH EYES CAUSES IRRITATION. PROLONGED OR REPEATED CONTACT WITH SKIN MAY CAUSE IRRITATION. DUSTS, VAPORS, OR MISTS MAY IRRITATE RESPIRATORY PASSAGES.

VI. PHYSICAL DATA

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | 200°F |
| --- | --- |
| SPECIFIC GRAVITY | 1.004 |
| pH | 11.7 |
| VAPOR DENSITY | HEAVIER THAN AIR |
| PERCENT VOLATILE BY WEIGHT | 93% |
| EVAPORATION RATE | SLOWER THAN ETHYL ETHER |
| SOLUBILITY IN WATER | COMPLETE |

VII. REACTIVITY

CONDITIONS TO AVOID: ANIONIC MATERIALS, STRONG OXIDIZERS

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: CONTAIN ALL SPILLS AND LEAKS TO PREVENT DISCHARGE TO THE ENVIRONMENT. VENTILATE AREA. RECOVER LIQUID AND DISPOSE WASTE MATERIAL IN ACCORDANCE WITH ALL APPLICABLE FEDERAL, STATE, AND LOCAL LAWS.

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | PROVIDE ADEQUATE VENTILATION |
| --- | --- |
| RESPIRATORY PROTECTION: | IF VAPORS OR FUMES ARE PRESENT, USE A NIOSH APPROVED RESPIRATOR |
| PROTECTIVE GLOVES: | LATEX |
| EYE PROTECTION: | CHEMICAL GOGGLES OR FACESHIELD |
| PERSONAL HYGIENE: | DO NOT REUSE CONTAMINATED CLOTHING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | AVOID OVERHEATING OR FREEZING - STORE IN A COOL WELL VENTILATED AREA - SPILLED MATERIAL IS SLIPPERY - THIS PRODUCT IS CLASSIFIED AS NON- HAZARDOUS |
| --- | --- |
| DOT IDENTIFICATION NUMBER | N/A |
| SHIPPING DESCRIPTION | DISINFECTANT, NOI, ITEM # 57100, SUB 3 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.