| NFPA | HMIS |
| --- | --- |
| Health-0 Fire-2 Reac-0 | Health- 0 Fire-2 Reac-0 |

DATE OF ISSUE- 5 / 30 / 00

I. IDENTIFICATION AND EMERGENCY INFORMATION

| PRODUCT NAME: | SANI-CLOTH PLUS GERMICIDAL TOWELETTES |
| --- | --- |
| PRODUCT CODE: | 993 |
| CHEMICAL NAME: | MIXTURE |
| CAS NUMBER: |  |
| PRODUCT APPEARANCE AND ODOR: | APPLICATOR IMPREGNATED WITH CLEAR LIQUID WITH ALCOHOL |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| HAZARDOUS INGREDIENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| ISOPROPYL ALCOHOL | 67-63-0 | 14.85 |
| QUATERNARY AMMONIUM CMPD | 139-0802 27479-28-3 | <1 |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | MAY CAUSE IRRITATION. FLUSH WITH COOL WATER FOR 15 MINUTES. |
| --- | --- |
| SKIN CONTACT | MAY CAUSE IRRITATION. IF RASH OR IRRITATION DEVELOPS, DISCONTINUE USE. |
| INHALATION | NOT NORMAL ROUTE OF ENTRY. REMOVE TO FRESH AIR. |
| INGESTION | NOT NORMAL ROUTE OF ENTRY. CONSULT PHYSICIAN. |

IV. FIRE AND EXPLOSION DATA

| FLASH POINT (MINIMUM): | 103oF |
| --- | --- |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | WATER, FOAM |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | NONE KNOWN |

V. PHYSICAL DATA

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | 181oF |
| --- | --- |
| SPECIFIC GRAVITY | 0.983 |
| SOLUBILITY IN WATER @ 1 ATM AND 25C | COMPLETE |

VI. REACTIVITY

| STABILITY: | STABLE |
| --- | --- |
| CONDITIONS TO AVOID: | NONE |
| INCOMPATIBILITY: | ANIONIC SURFACTANT, SOAPS |
| HAZARDOUS DECOMPOSITION: | NONE KNOWN |
| POLYMERIZATION: | WILL NOT OCCUR |

VIII. SPILL OR LEAK PROCEDURES

| SPILL CONTROL: | CONTAIN WITH ABSORBENT MATERIAL. REMOVE IGNITION SOURCES. |
| --- | --- |
| WASTE DISPOSAL METHOD: | TREAT AS FLAMMABLE LIQUID. FOLLOW LOCAL, STATE AND FEDERAL REGULATIONS. |
| HANDLING AND STORAGE: | STORE AWAY FROM HEAT AND IGNITION SOURCES. |

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | RECOMMENDED |
| --- | --- |
| RESPIRATORY PROTECTION: | USE NIOSH APPROVED RESPIRATOR IF EXPOSURE LEVEL IS EXCEEDED. |
| PROTECTIVE GLOVES: | RECOMMENDED |
| EYE PROTECTION: | IF SPLASHING MAY OCCUR. |
| OTHER PROTECTIVE EQUIPMENT: | NONE |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | N/A |
| --- | --- |
| DOT IDENTIFICATION NUMBER | N/A |
| SHIPPING DESCRIPTION |  |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.