| NFPA | HMIS |
| --- | --- |
| Health-3 Fire-0 Reac-1 | Health-3 Fire-0 Reac-1 |
| DATE OF ISSUE 6/1/00 |  |
| I. IDENTIFICATION AND EMERGENCY INFORMATION |  |

| PRODUCT NAME: | pH CONTROL® |
| --- | --- |
| PRODUCT CODE: | 062 |
| CHEMICAL NAME: | SODIUM HYDROXIDE |
| CAS NUMBER: | 1310-73-2 |
| PRODUCT APPEARANCE AND ODOR: | CLEAR TO SLIGHTLY CLOUDY - NO ODOR |
| EMERGENCY TELEPHONE NUMBERS: CHEMTREC IN DISTRICT OF COLUMBIA OUTSIDE THE CONTINENTAL U.S. | FOR CHEMICAL EMERGENCY 1-800-424-9300 COLLECT 483-7616 COLLECT (202) 483-7616 |

II. COMPONENTS AND HAZARD INFORMATION

| COMPONENTS | CAS NO. OF COMPONENTS | APPROXIMATE CONCENTRATION |
| --- | --- | --- |
| SODIUM HYDROXIDE | 1310-73-2 | 20% |

| EXPOSURE LIMIT FOR TOTAL PRODUCT | 2 mg/m³ |
| --- | --- |
| BASIS | TLV |

III. EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | FLUSH WITH WATER FOR 15 MINUTES -GET MEDICAL ATTENTION |
| --- | --- |
| SKIN CONTACT | FLUSH WITH WATER FOR 15 MINUTES -GET MEDICAL ATTENTION |
| INHALATION | MOVE TO FRESH AIR - TREAT SYMPTOMATICALLY - GET MEDICAL ATTENTION |
| INGESTION | DO NOT INDUCE VOMITING - DRINK LARGE QUANTITIES OF MILK OR WATER - SEEK MEDICAL ATTENTION |
| IV. FIRE AND EXPLOSION DATA |  |

| FLASH POINT (MINIMUM): | NON-FLAMMABLE |
| --- | --- |
| AUTOINGNITION TEMPERATURE: | NOT LISTED F NOT LISTED C |
| HANDLING PRECAUTIONS: | DO NOT STORE NEAR COMBUSTIBLE OR ORGANIC CHEMICALS WITH WHICH IT CAN REACT |
| FLAMMABLE OR EXPLOSIVE LIMITS (APPROX. % BY VOLUME IN AIR): | NON-FLAMMABLE |
| EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES: | WATER SPRAY, FOAM, CARBON DIOXIDE, OR DRY CHEMICALS - USE FULL PROTECTIVE CLOTHING AND SELF CONTAINED BREATHING APPARATUS |
| DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: | WOULD PROBABLY EVOLVE SODIUM MONOXIDE |
| “EMPTY” CONTAINER WARNINGS | RINSE THOROUGHLY WITH WATER - DISPOSE OF CONTAINER IN CONFORMANCE WITH FEDERAL, STATE, AND LOCAL REGULATIONS |

V. HEALTH AND HAZARD INFORMATION

| VARIABILITY AMONG INDIVIDUALS: | NONE KNOWN |
| --- | --- |
| EFFECTS OF OVEREXPOSURE: (SIGNS AND SYMPTOMS OF EXPOSURE | MOMENTARY STINGING OF EYE, NOSE AND THROAT TISSUE |

NATURE OF HAZARD AND TOXICITY INFORMATION:

THIS COMPOUND IS EXTREMELY ALKALINE IN NATURE AND IS CORROSIVE TO BODY TISSUE - DERMATITIS MAY RESULT FROM REPEATED EXPOSURE TO SOLUTIONS IN THE FORM OF LIQUIDS OR MISTS

VI. PHYSICAL DATA

THE FOLLOWING DATA ARE APPROXIMATE OR  TYPICAL VALUES AND SHOULD  NOT BE USED FOR PRECISE DESIGN PURPOSES.

| BOILING POINT | 102°C |
| --- | --- |
| SPECIFIC GRAVITY | 1.21 |
| pH | 12.5 |
| PERCENT VOLATILE BY WEIGHT | 80%@ 100 o C / 2 HOURS |
| EVAPORATION RATE @ 1 ATM AND 25°C | LIKE WATER |
| SOLUBILITY IN WATER @ 1 ATM AND 25°C | COMPLETE |

VII. REACTIVITY

NON COMBUSTIBLE - STABLE - IF WATER IS DRIVEN OFF, SOLID SODIUM HYDROXIDE WILL REACT VIOLENTLY WITH WATER AND GENERATE SUFFICIENT HEAT TO IGNITE NEARBY COMBUSTIBLE

VIII. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS SPILLED OR RELEASED: DILUTE WITH LARGE VOLUMES OF WATER AND FLUSH DOWN THE DRAIN IN ACCORDANCE WITH FEDERAL, STATE, AND LOCAL LAWS - IF NEUTRALIZATION IS REQUIRED, USE MILD ACIDS, PREFERABLY ACETIC ACID

IX. PROTECTION AND PRECAUTIONS

| VENTILATION: | LOCAL EXHAUST AT POINT TO TRANSFER TO REDUCE LEVEL TO LESS THAN 2 mg/m³ |
| --- | --- |
| RESPIRATORY PROTECTION: | NOT NEEDED IF LOCAL EXHAUST IS SATISFACTORY - IF NOT USE MESA - NIOSH APPROVED RESPIRATOR FOR AEROSOLS |
| PROTECTIVE GLOVES: | RUBBER, VINYL OR NEOPRENE |
| EYE PROTECTION: | CHEMICAL WORK GOGGLES - FULL FACE SHIELD IF SPLASHING IS POSSIBLE |
| OTHER PROTECTIVE EQUIPMENT: | RUBBER APRON IF SPLASHING IS POSSIBLE |
| PERSONAL HYGIENE: | WASH AFTER HANDLING - WASH CONTAMINATED GARMENTS BEFORE REUSING |

X. TRANSPORTATION INFORMATION

| TRANSPORTATION INCIDENT INFORMATION: | CLASSIFIED AS A CORROSIVE - NEUTRALIZE WITH DILUTED MILD ACID - FLUSH WITH WATER |
| --- | --- |
| DOT IDENTIFICATION NUMBER | UN 1824 |
| SHIPPING DESCRIPTION | SODIUM HYDROXIDE SOLUTION, 8, UN 1824, PG II, ITEM # 48580, SUB 3 |

THE INFORMATION AND RECOMMENDATIONS CONTAINED HEREIN ARE, TO THE BEST OF PHARMACAL’S KNOWLEDGE AND BELIEF, ACCURATE AND RELIABLE AS OF THE DATE ISSUED. PHARMACAL DOES NOT WARRANT OR GUARANTEE THEIR ACCURACY OR RELIABILITY, AND PHARMACAL SHALL NOT BE LIABLE FOR ANY LOSS OR DAMAGE ARISING OUT OF THERE USE THEREOF.

THE INFORMATION AND RECOMMENDATIONS ARE OFFERED FOR THE USER’S CONSIDERATION AND EXAMINATION, AND IT IS THE USER’S RESPONSIBILITY TO SATISFY ITSELF THAT THEY ARE SUITABLE AND COMPLETE FOR ITS PARTICULAR USE.

THE HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) AND NATIONAL FIRE PROTECTION ASSOCIATION RATINGS HAVE BEEN INCLUDED BY PHARMACAL RESEARCH LABORATORIES INC. IN ORDER TO PROVIDE ADDITIONAL HEALTH AND HAZARD INFORMATION. THE RATINGS RECOMMENDED ARE BASED UPON CRITERIA SUPPLIED BY THE DEVELOPERS OF THESE RATING SYSTEMS, TOGETHER WITH PHARMACAL’S INTERPRETATION OF THE AVAILABLE DATA.